	[image: image12.jpg]UAB ,AluFSolutions “
Company code: 305428762
VAT code: LT100012946115

Pramonés str. 19, Parapijoniskés, Vilnius dist.

LT-13241, Lithuania

Bank: Luminor Bank AS
Bank code: 304870069
Acc. No.: LT734010051005201968
SWIFT code: AGBLLT2X

Partner

AluFSolutions



 
	 
	 
	 
	 
	 
	 
	 
	Date:

	 
	 
	 
	 
	 
	 
	 
	 
	2022-02-28

	
	

	

	
	

	
	

	
	

	Item no.
	Number
	Description
	UP
	TP

	L1
	2
	
	
	
	

	[image: image1.jpg]Outside
DIN EN 12519

R
<

'

1000

2500





	Profile system: Schüco AWS 90.SI+/ADS 90.SI/ADS 90 PL.SI

1-part Aluminium unit

Unit dimension [W x H]: 1000 mm x 2530 mm

Surface finish Special colours 2: Tiger 9005 Coatex (or analog) Jet black

1 Number TT DIN right window

Vent size [W x H]: 942 mm x 2442 mm

Fittings type: BASIC - AvanTec SimplySmart 130-kg fitting; Type 2 (tall unit, additional vertical locking point); Stay 400

Closing assistance: Spring-loaded unit, size 2

Watertightness: 9A

Limiting stay: Rebate (90° opening angle)

Handle inside: Push w-hdle 9005 IW Handle position: 1221 mm (Centre)

Glass: 4TF/20arg/4/20arg/4ESGTF+Multitech , Ug value: 0,5 W/(m²K)

Inclusive:

Base profile

Measurement value Uw,BW: 0,74 W/(m²K)

	 

	
	

	L2
	2
	
	
	
	

	[image: image2.jpg]Outside
DIN EN 12519

2500





	Profile system: Schüco AWS 90.SI+/ADS 90.SI/ADS 90 PL.SI

1-part Aluminium unit

Unit dimension [W x H]: 1000 mm x 2530 mm

Surface finish Special colours 2: Tiger 9005 Coatex (or analog) Jet black

1 Number TT DIN left window

Vent size [W x H]: 942 mm x 2442 mm

Fittings type: BASIC - AvanTec SimplySmart 130-kg fitting; Type 2 (tall unit, additional vertical locking point); Stay 400

Closing assistance: Spring-loaded unit, size 2

Watertightness: 9A

Limiting stay: Rebate (90° opening angle)

Handle inside: Push w-hdle 9005 IW Handle position: 1221 mm (Centre)

Glass: 4TF/20arg/4/20arg/4ESGTF+Multitech , Ug value: 0,5 W/(m²K)

Inclusive:

Base profile

Measurement value Uw,BW: 0,74 W/(m²K)

	 

	
	

	
	

	
	

	
	

	L3
	1
	
	
	
	

	[image: image3.jpg]Outside
DIN EN 12519

2500





	Profile system: Schüco ASE 80.HI

1-part Aluminium unit

Unit dimension [W x H]: 4195 mm x 2550,8 mm

Surface finish Special colours 2: Tiger 9005 Coatex (or analog) Jet black

1 Number Type 3E/1

Vent 1 [W x H]: 1413 mm x 2401 mm (b1: 1422 mm)

Vent 2 [W x H]: 1413 mm x 2401 mm (b2: 1351 mm)

Vent 3 [W x H]: 1413 mm x 2401 mm (b3: 1422 mm)

Fittings type: Standard 200-kg fitting, 3-point locking, 1007 mm Griffhöhe

Handle inside: Lift/slide hndle P cy 9005 (Handle position: 1007 mm)

Handle outside: Handle rec. 9005 (Handle position: 1007 mm)

Glass: 6ESGTF/18arg/6/16arg/6ESGTF+Multitech , Ug value: 0,5 W/(m²K)

Inclusive:

Drip bar

Base profile

Measurement value Uw,BW: 0,88 W/(m²K)

	 

	
	


	[image: image13.jpg]UAB ,AluFSolutions “
Company code: 305428762
VAT code: LT100012946115

Pramonés str. 19, Parapijoniskés, Vilnius dist.

LT-13241, Lithuania

Bank: Luminor Bank AS
Bank code: 304870069
Acc. No.: LT734010051005201968
SWIFT code: AGBLLT2X

Partner

AluFSolutions



L4
	1
	
	
	
	

	[image: image4.jpg]|
|4

Outside
DIN EN 12519

14
<

2500

i<
<
1550





	Profile system: Schüco AWS 90.SI+/ADS 90.SI/ADS 90 PL.SI

1-part Aluminium unit

Unit dimension [W x H]: 1550 mm x 2530 mm

Surface finish Special colours 2: Tiger 9005 Coatex (or analog) Jet black

1 Number Fixed glazing (Glass: 6ESGTF/18arg/6/16arg/6ESGTF+Multitech , Ug value: 0,5 W/(m²K))

Inclusive:

Base profile

Measurement value Uw,BW: 0,62 W/(m²K)

	 

	
	

	L5
	1
	
	
	
	

	[image: image5.jpg]Outside
DIN EN 12519

14
<

'
lq
800

2500





	Profile system: Schüco AWS 90.SI+/ADS 90.SI/ADS 90 PL.SI

1-part Aluminium unit

Unit dimension [W x H]: 800 mm x 2530 mm

Surface finish Special colours 2: Tiger 9005 Coatex (or analog) Jet black

1 Number Fixed glazing (Glass: 6ESGTF/18arg/6/16arg/6ESGTF+Multitech , Ug value: 0,5 W/(m²K))

Inclusive:

Base profile

Measurement value Uw,BW: 0,69 W/(m²K)

	 

	
	

	L6
	1
	
	
	
	

	[image: image6.jpg]<

|

2

|

|

|

|

|

! (o]
BY R .- 1 --- - - - 2-- - - - - - 3 - AN

| o

|

|

|

|

J

e

|

13693 1369,3 13693
4108

Outside

DIN EN 12519





	Profile system: Schüco AWS 90.SI+/ADS 90.SI/ADS 90 PL.SI

3-part Aluminium unit

Unit dimension [W x H]: 4108 mm x 2530 mm

Surface finish Special colours 2: Tiger 9005 Coatex (or analog) Jet black

3 Number Fixed glazing (Glass: 6ESGTF/18arg/6/16arg/6ESGTF+Multitech , Ug value: 0,5 W/(m²K))

Inclusive:

Base profile

Measurement value Uw,BW: 0,63 W/(m²K)

	 

	
	


	[image: image14.jpg]UAB ,AluFSolutions “
Company code: 305428762
VAT code: LT100012946115

Pramonés str. 19, Parapijoniskés, Vilnius dist.

LT-13241, Lithuania

Bank: Luminor Bank AS
Bank code: 304870069
Acc. No.: LT734010051005201968
SWIFT code: AGBLLT2X

Partner

AluFSolutions



L7
	1
	
	
	
	

	[image: image7.jpg]Outside
DIN EN 12519

2500





	Profile system: Schüco ASE 80.HI

1-part Aluminium unit

Unit dimension [W x H]: 4209 mm x 2550,8 mm

Surface finish Special colours 2: Tiger 9005 Coatex (or analog) Jet black

1 Number Type 3E/1.2

Vent 1 [W x H]: 1417,7 mm x 2401 mm (b1: 1426,7 mm)

Vent 2 [W x H]: 1417,7 mm x 2401 mm (b2: 1355,7 mm)

Vent 3 [W x H]: 1417,7 mm x 2401 mm (b3: 1426,7 mm)

Fittings type: Standard 200-kg fitting, 3-point locking, 1007 mm Griffhöhe

Handle inside: Lift/slide hndle P cy 9005 (Handle position: 1007 mm)

Handle outside: Handle rec. 9005 (Handle position: 1007 mm)

Glass: 6ESGTF/18arg/6/16arg/6ESGTF+Multitech , Ug value: 0,5 W/(m²K)

Inclusive:

Drip bar

Base profile

Measurement value Uw,BW: 0,88 W/(m²K)

	 

	
	

	L8
	1
	
	
	
	

	[image: image8.jpg]9804

Outside
DIN EN 12519




	Profile system: Schüco ASE 80.HI

1-part Aluminium unit

Unit dimension [W x H]: 9804 mm x 2550,8 mm

Surface finish Special colours 2: Tiger 9005 Coatex (or analog) Jet black

1 Number Type 3F.A

Vent 1 [W x H]: 1667,3 mm x 2401 mm (b1: 1671,3 mm)

Vent 2 [W x H]: 1667,3 mm x 2401 mm (b2: 1595,3 mm)

Vent 3 [W x H]: 1667,3 mm x 2401 mm (b3: 1635,3 mm)

Vent 4 [W x H]: 1667,3 mm x 2401 mm (b4: 1635,3 mm)

Vent 5 [W x H]: 1667,3 mm x 2401 mm (b5: 1595,3 mm)

Vent 6 [W x H]: 1667,3 mm x 2401 mm (b6: 1671,3 mm)

Fittings type: Standard 350-kg fitting, 3-point locking, 1008 mm Griffhöhe

Handle inside: Lift/slide hndle P cy 9005 (Handle position: 1008 mm)

Handle outside: Handle rec. 9005 (Handle position: 1008 mm)

Glass: 6ESGTF/18arg/6/16arg/6ESGTF+Multitech , Ug value: 0,5 W/(m²K)

Inclusive:

Drip bar

Base profile

Measurement value Uw,BW: 0,85 W/(m²K)

	 

	
	

	LD1
	1
	
	
	
	

	[image: image9.jpg]Outside
DIN EN 12519

14
<

‘<
<

1100

2500





	Profile system: Schüco AWS 90.SI+/ADS 90.SI/ADS 90 PL.SI

1-part Aluminium unit

Unit dimension [W x H]: 1100 mm x 2530 mm

Surface finish Special colours 2: Tiger 9005 Coatex (or analog) Jet black

1 Number Single leaf door DIN LH Schüco ADS Inward-opening

Vent size [W x H]: 992 mm x 2438 mm

Fittings type: Standard security

With heavy loading

Fittings colour: Jet black

Lock: 3-point claw bolt and pin lock 39 mm

Cylinder: Thumb trn cy. 75/30

Rose: PC rosette Oval 9005, PC rosette Oval 9005

Door hinges: 3 Number Conc-hg 180

Door air space: 8 mm

Handle, inside: Door hndle Strght 9005 (Handle position: 1050 mm)

Handle, outside: Door hndle Strght 9005 (Handle position: 1050 mm)

Glass: 4ESGTF/20arg/4/20arg/4ESGTF+Multitech , Ug value: 0,5 W/(m²K)

Inclusive:

Base profile

Measurement value Uw,BW: 1,1 W/(m²K)

	 

	
	


	[image: image15.jpg]UAB ,AluFSolutions “
Company code: 305428762
VAT code: LT100012946115

Pramonés str. 19, Parapijoniskés, Vilnius dist.

LT-13241, Lithuania

Bank: Luminor Bank AS
Bank code: 304870069
Acc. No.: LT734010051005201968
SWIFT code: AGBLLT2X

Partner

AluFSolutions



P1
	1
	
	
	
	

	[image: image10.jpg]|®
14

Outside
DIN EN 12519

2500

900

1800





	Profile system: Schüco AWS/ADS 65

2-part Aluminium unit

Unit dimension [W x H]: 1800 mm x 2530 mm

Surface finish Special colours 2: Tiger 9005 Coatex (or analog) Jet black

2 Number Fixed glazing (Glass: 4ESG/20arg/4ESGTF+Chrm.Ultra , Ug value: 1,1 W/(m²K))

Inclusive:

Base profile

Measurement value Uw,BW: 1,4 W/(m²K)

	 

	
	

	P2
	1
	
	
	
	

	[image: image11.jpg]Outside '<
DIN EN 12519 1800

2500





	Profile system: Schüco AWS/ADS 65

1-part Aluminium unit

Unit dimension [W x H]: 1800 mm x 2530 mm

Surface finish Special colours 2: Tiger 9005 Coatex (or analog) Jet black

1 Number Double leaf door DIN LH Schüco ADS HD Inward-opening

Vent 1 [W x H]: 861 mm x 2443 mm

Vent 2 [W x H]: 861 mm x 2443 mm

Fittings type: Standard security

With heavy loading

Fittings colour: Jet black

Lock: 3-point swing bolt lock 34 mm

Cylinder: Double cylindr 50/30 30°

Shoot bolt lock: DRIVER BAR 40MM

Rose: PC rosette Oval 9005, PC rosette Oval 9005

Door hinges: 4 Number Brrl hng R01 RAL9005, 4 Number Brrl hng R01 RAL9005

Door air space: 8 mm

Handle, inside (access leaf): Door hndle Strght 9005 (Handle position: 1050 mm)

Handle, outside (access leaf): Door hndle Strght 9005 (Handle position: 1050 mm)

Glass: 4ESG/20arg/4ESGTF+Chrm.Ultra , Ug value: 1,1 W/(m²K)

Inclusive:

Base profile

Measurement value Uw,BW: 1,9 W/(m²K)

	 

	
	

	

	
	
	
	

	
	
	
	

	

	

	

	
	
	
	

	

	
	
	
	

	

	
	

	
	


	[image: image16.jpg]UAB ,AluFSolutions “
Company code: 305428762
VAT code: LT100012946115

Pramonés str. 19, Parapijoniskés, Vilnius dist.

LT-13241, Lithuania

Bank: Luminor Bank AS
Bank code: 304870069
Acc. No.: LT734010051005201968
SWIFT code: AGBLLT2X

Partner

AluFSolutions




	

	

	
	


�





�





�





�





�








